# RGB And Dimmable Lighting Demo

# Hardware Environment

- MIMXRT1050-EVKB

- DR1175 1V2 Light Module

- Cypress WiFi Module – J1’s pin 2 and 3 should be short connected with a jumper, leave other pins opened.

# Software Environment

- SDK – linkit_demo.

- IDE – IAR Embedded Workbench for ARM – v8.32.4.

- Android App for ALI platform:

# SDK Porting Settings

This step already done, there is no need to do the porting work unless you want to do the porting work again.

# SDK Settings

  - Turn off the data co-processor (DCP) feature by change settings of MIMXRT1052_features.h:

#define FSL_FEATURE_SOC_DCP_COUNT (0)

  - MBEDTLS_CONFIG_FILE="ksdk_mbedtls_config.h"

  - PRINTF_ADVANCED_ENABLE=1

  - PRINTF_FLOAT_ENABLE=1

# CSDK Extract Settings

  - PLATFORM_HAS_STDINT

  - PLATFORM_HAS_DYNMEM

  - PLATFORM_HAS_OS enabled to use Free-RTOS

  - FEATURE_INFRA_NETWORK_PAYLOAD

  - FEATURE_INFRA_LOG

  - FEATURE_MQTT_COMM_ENABLED

    - FEATURE_MQTT_DEFAULT_IMPL

    - FEATURE_MQTT_DIRECT

  - DEVICE_MODEL_ENABLED

    - DEVICE_MODEL_SHADOW

  - SUPPORT_TLS enabled to use mbedtls to secure the communication

  - DEV_BIND_ENABLED enabled to let app bind with device through CoAP messages.

Setup Step

- Download the linkit demo SDK from below link:

https://github.com/junexunxp/linkit_demo_cypress

- Open file ../ linkit_demo_cypress/linkit_demo.eww, the SDK file structures listed as below:

- Open file source/app_preinclude.h, change the macro definition DEMO_OPTION to DEMO_DIM_LIGHT to run the dimmable light demo, or change DEMO_OPTION to DEMO_RGB_LIGHT to run the RGB light demo.

- From IAR’s left top corner select compile option “flexspi_nor_debug”, then make the project.

- Light module DR1175 connected to IMXRT1050 EVKB by using the Arduino interface as below:

- Download compiled image to the EVKB and reset the board.

- Connect IMXRT1050 EVKB’s J9 micro USB port to PC, and open a serial port communication tool, e.g.“putty”, use the shell command to configure the WiFi security materials to 1050 EVKB, the command use: “iwconfig wlan0 ssid iPhone key 12345678”.

It will connect to AP “iPhone” with key “12345678”.

- Connect your Android phone to the same AP as step 7, open Ali Cloud App, press the “+” button on the right top corner, now you can see the device and add it.

- The demo results as below.