Smart Washing Machine Demo

# Hardware Environment

- MIMXRT1050-EVKB

- RK043FN02H-CT LCD

- Cypress WiFi Module – J1’s pin 2 and 3 should be short connected with a jumper, leave other pins opened.

# Software Environment

- SDK – linkit_demo.

- IDE – IAR Embedded Workbench for ARM – v8.32.4.

- Android App for ALI platform:

# CSDK Extract Settings

  - PLATFORM_HAS_STDINT

  - PLATFORM_HAS_DYNMEM

  - PLATFORM_HAS_OS enabled to use Free-RTOS

  - FEATURE_INFRA_NETWORK_PAYLOAD

  - FEATURE_INFRA_LOG

  - FEATURE_MQTT_COMM_ENABLED

    - FEATURE_MQTT_DEFAULT_IMPL

    - FEATURE_MQTT_DIRECT

  - DEVICE_MODEL_ENABLED

    - DEVICE_MODEL_SHADOW

  - SUPPORT_TLS enabled to use mbedtls to secure the communication

  - DEV_BIND_ENABLED enabled to let app bind with device through CoAP messages.

# Setup Step

- Download the linkit demo SDK from below link:

https://github.com/junexunxp/linkit_demo_cypress

- Open file ../ linkit_demo_cypress/smart_washing_machine.eww, the SDK file structures listed as below:

- To run the smart washing machine demo, open file source/app_preinclude.h, change the macro definition DEMO_OPTION to DEMO_WASHING_MACHINE

- From IAR’s left top corner select compile option “flexspi_nor_debug”, then make the project.

- The hardware should be connected as below:

Front

Back

- Download compiled image to the EVKB and reset the board.

- Connect IMXRT1050 EVKB’s J9 micro USB port to PC, and open a serial port communication tool, e.g.“putty”, use the shell command to configure the WiFi security materials to 1050 EVKB, the command use: “iwconfig wlan0 ssid iPhone key 12345678”.

It will connect to AP “iPhone” with key “12345678”.

- Connect your Android phone to the same AP as step 7, open Ali Cloud App, press the “+” button on the right top corner, now you can see the device ”smart_washing_machine…”, and add it.

- By changing the attributes from app, the settings will be automatically synchronized to the GUI. And same inverse.